Appendix A. Scoring Methodology for All Attributes

| Objective | Attribute | Measurement Scales | Score | References and Notes |
| --- | --- | --- | --- | --- |
| Access Difficulty | Timescale to travela (yrs.) | 0 – 1 month | 5 | aScoring methodology assigned by the author based on comparative analysis of attribute relevance. |
|  |  | 1 month – 12 months | 4 |  |
|  |  | 12 months – 18 months | 3 |  |
|  |  | 18- 24 months | 2 |  |
|  |  | 2 - 3 years | 1 |  |
|  |  | 3+ years | 0 |  |
|  | Amount of fuel neededb (kg) | <500 kg | 5 | bScoring methodology assigned by the author based on comparative analysis of attribute relevance. |
|  |  | 500 – 1500 kg | 4 |  |
|  |  | 1500 – 5,000 kg | 3 |  |
|  |  | 5,000 – 10,000 kg | 2 |  |
|  |  | 10,000+ kg | 1 |  |
|  | Orbital entry ∆v requiredc | 0 – 250 m/s | 5 | ccmglee, 2024 |
|  |  | 250 – 500 m/s | 4 |  |
|  |  | 500 – 1000 m/s | 3 |  |
|  |  | 1000 – 3000 m/s | 2 |  |
|  |  | 3000+ m/s | 1 |  |
|  | Orbit to landing ∆v requiredc | 0 – 1000 m/s | 5 | ccmglee, 2024 |
|  |  | 1000 – 2000 m/s | 4 |  |
|  |  | 2000 – 3000 m/s | 3 |  |
|  |  | 3000 – 4000 m/s | 2 |  |
|  |  | 4000+ m/s | 1 |  |
|  | Access to other bodies in the system from main sited Timescale to travela (yrs.) | Many other bodies of interest easily accessible by humans or a body with significant resources | 5 | dRoyal Museums Greenwich, 2025 |
|  |  | Many other bodies of interest nearby accessible by robotic methods and with some difficulty for humans | 4 |  |
|  |  | Few or one other body of interest accessible by human or robotic methods | 3 |  |
|  |  | 0 – 1 month | 5 |  |
|  |  | 1 month – 12 months | 4 |  |
|  |  | 12 months – 18 months | 3 |  |
| Environ-mental Challenges | Gravitational Health Effects | Near 1g up to 1ge | 5 | eJones, 2021. |
|  |  | 0.4g to near 1ge | 4 |  |
|  |  | 0.25g to 0.4gf | 3 | fGoswami, 2021 |
|  |  | Near lunar g to 0.25gf | 2 |  |
|  |  | 0g to Near lunar gg | 1 | gNguyen & Kim, 2020 |
|  | Radiation Health Effectsh | Safe for human physiology | 5 | hHealth and Physics Society, 2025 |
|  |  | Cancer risks increased. Heightened risk of cataracts. | 4 |  |
|  |  | Cancer risks increased. Observable health effects. | 3 |  |
|  |  | Nausea, vomiting and fatigue with some hair loss. Very high cancer rate. | 2 |  |
|  |  | Serious health effects, death for half the population, skin burning. | 1 |  |
|  |  | Deadly, no survivors | 0 |  |
|  | Extreme Cold Engineering Hazards | Favorable Coldi | 5 | Buckley, 2024i |
|  |  | Cold, but Habitablei | 4 |  |
|  |  | Harsh but Manageablej | 3 | Kittel & McKuen, 2018j |
|  |  | Severe Cold Stressj | 2 |  |
|  |  | Extreme Cryogenic Instabilityk | 1 | Enss & Hunklinger, 2005k |
|  | Communications Delayl | Instantaneous to a few seconds | 5 | lNASA-JPL, 2025 for astronomic distances from sun and calculations for speed of light travel of a communications signal |
|  |  | Tens of seconds | 4 |  |
|  |  | Tens of minutes | 3 |  |
|  |  | One to a few hours | 2 |  |
|  |  | Multiple Hours to Days | 1 |  |
|  |  | Days to weeks | 0 |  |
|  | Seismic/Volcanic Hazardsm | Rare or not present (Most moons in the solar system) | 3 | mScoring methodology assigned by the author based on comparative analysis of attribute relevance |
|  |  | Occasional – Mitigation usually or often required (Mars and Titan) | 2 |  |
|  |  | Common – Mitigation required (Earth) | 1 |  |
|  |  | Extreme – Not possible for human presence | 0 |  |
|  | Weather Hazardsn | Rare or not present (Most moons in the solar system) | 3 | nScoring methodology assigned by the author based on comparative analysis of attribute relevance |
|  |  | Occasional – Mitigation usually or often required (Mars and Titan) | 2 |  |
|  |  | Common – Mitigation required (Earth) | 1 |  |
|  |  | Extreme – Not possible for human presence | 0 |  |
|  | Meteoroid Impactso | Strong atmospheric protection – mitigation not required (Earth or Titan) | 3 | oScoring methodology assigned by the author based on comparative analysis of attribute relevance |
|  |  | Light atmospheric protection – mitigation suggested (Mars) | 2 |  |
|  |  | No atmospheric protection – mitigation required (airless bodies) | 1 |  |
|  |  | Too dangerous for human or robotic activity (early solar system) | 0 |  |
|  | Visibility/Lighting conditionsp | Significant positive effects on SAD | 5 | pIlluminating Engineering Society of North America, 2011 |
|  |  | Improved alertness and mood | 4 |  |
|  |  | Lighting not required during daytime periods | 3 |  |
|  |  | Lighting of sufficient value for normal work conditions. Mood may be reduced with long exposures | 2 |  |
|  |  | May lead to eye strain and under-stimulation, contributing to poorer mood and cognitive performance during long periods | 1 |  |
|  |  | Completely Opaque, artificial lighting of limited value (bottom of Earth’s oceans) | 0 |  |
|  | Terrain Challengesq | Smooth or mixed with infrastructure in place to enable easy travel (Earth) | 5 | qScoring methodology assigned by the author based on comparative analysis of attribute relevance |
|  |  | Mixed – smooth and rough terrain but other modes available (atmospheric or oceanic) | 4 |  |
|  |  | Mixed – smooth and rough terrain | 3 |  |
|  |  | Rough - Adequate for rovers with addition capabilities | 2 |  |
|  |  | Extremely rough – Specially equipped vehicle required (tank, monster truck, etc.) | 1 |  |
|  |  | Jagged terrain - (cliffs, steep inclines, soft dunes, etc.) Surface movement not possible | 0 |  |
|  | Regolith Hazardsr | None – Benign environment | 5 | rScoring methodology assigned by the author based on comparative analysis of attribute relevance |
|  |  | Low – minor hazards that can be tolerated | 4 |  |
|  |  | Moderate – Hazards present but manageable | 3 |  |
|  |  | High – Considerable risks that require mitigation | 2 |  |
|  |  | Very High – significant mitigation required | 1 |  |
|  |  | Extreme – insurmountable challenges | 0 |  |
| Economic / Resource or Sustain-ability Benefits | Mineral resources for Earths | Rich ore content (Earth) | 5 | sScoring methodology assigned by the author based on comparative analysis of attribute relevance |
|  |  | High ore content (asteroid belt) | 4 |  |
|  |  | Moderate ore content (Moon) | 3 |  |
|  |  | Low ore content (outer solar system) | 2 |  |
|  |  | Minimal ore content (far outer solar system) | 1 |  |
|  |  | Negligible (gas giants), such as Jupiter | 0 |  |
|  | Water Availabilityt | Exceptionally High-water content (Earth), easily extractable | 5 | tScoring methodology assigned by the author based on comparative analysis of attribute relevance |
|  |  | High water abundance, relatively accessible (Ceres) | 4 |  |
|  |  | Moderate Water available, extraction feasible but requires energy | 3 |  |
|  |  | Modest water available but with extraction hurdles (Moon) | 2 |  |
|  |  | Minimal water with extreme extraction difficulty | 1 |  |
|  |  | Negligible or inaccessible | 0 |  |
|  | Atmospheric resource availability (N2 and O2)u | Exceptionally rich and easily processed | 5 | uScoring methodology assigned by the author based on comparative analysis of attribute relevance |
|  |  | Abundant for one component and generally favorable extraction | 4 |  |
|  |  | Moderate availability with feasible but complex extraction | 3 |  |
|  |  | Limited availability with significant processing needs | 2 |  |
|  |  | Trace availability with extreme challenges | 1 |  |
|  |  | None to negligible available | 0 |  |
|  | DFD Fusion fuel availability (Deuterium and Helium3)v | Abundant and easily extractable | 4 | vScoring methodology assigned by the author based on comparative analysis of attribute relevance |
|  |  | Good to Abundant, but not easily extractable fuel reserves | 3 |  |
|  |  | Moderate with viable fuel presence, or good to abundant availability but with major challenges to extraction | 2 |  |
|  |  | Low fuel availability or severe technical challenges to extraction | 1 |  |
|  |  | Negligible or practically inaccessible fuel with extremely difficult challenges to extract | 0 |  |
|  | Alternative Fusion Resourcesw, x -Deuterium-Tritium (D-T) -Deuterium-Deuterium (D-D) -Proton-Boron11 (p-B11) | Very High (Earth) | 5 | wSzlugaj, & Radwanek-Bąk, 2022 for Lithium-D-T, and D-D; xNagar, 2022, for Boron availability |
|  |  | High | 4 |  |
|  |  | Moderate | 3 |  |
|  |  | Low-to-moderate | 2 |  |
|  |  | Very low | 1 |  |
|  |  | Negligible | 0 |  |
|  | Building materialsy | Exceptional - resources available in large quantities with minimal processing to yield good quality construction materials | 5 | yScoring methodology assigned by the author based on comparative analysis of attribute relevance |
|  |  | Abundant -mechanically robust materials that are relatively easily processed | 4 |  |
|  |  | Moderate - regolith (or surface rock) can be processed into building materials, but additional refinement is needed to achieve decent quality | 3 |  |
|  |  | Limited - might be mixtures of ice and rock or loosely consolidated materials. Less than ideal applications even after significant processing | 2 |  |
|  |  | Materials present but consist mainly of volatile ices that require extensive processing for load-bearing structural elements | 1 |  |
|  |  | No materials available that are accessible | 0 |  |
|  | Complex chemical resourcesz | Exceptionally rich and complex chemistry | 5 | zScoring methodology assigned by the author based on comparative analysis of attribute relevance |
|  |  | High levels of organics with a significant diversity of carbon-based compounds | 4 |  |
|  |  | Moderate presence—organics are detectable with evidence for a range of molecules, though not in extreme abundance | 3 |  |
|  |  | Low-level organic chemistry exists with limited diversity or complexity, but overall production or accumulation is modest | 2 |  |
|  |  | Only trace amounts of simple chemistry (usually exogenous, e.g., delivered via micrometeorites) | 1 |  |
|  |  | Essentially no meaningful complex chemistry. | 0 |  |
|  | Radioactive elementsaa | Exceptionally rich in radioactive elements with minimal processing required | 5 | aaScoring methodology assigned by the author based on comparative analysis of attribute relevance |
|  |  | High concentrations of radioactive elements exist that are relatively easily extractable with minimal processing (Earth). | 4 |  |
|  |  | Moderate levels; somewhat difficult to extract | 3 |  |
|  |  | Low concentrations exist; extraction would be very challenging | 2 |  |
|  |  | Trace amounts detectable; requiring extensive processing | 1 |  |
|  |  | Virtually no accessible radioactive elements | 0 |  |
|  | Alternative Energyab | Solar Energy greater than 1000 W/m2 (inner solar system) | 5 | abScoring methodology assigned by the author based on comparative analysis of attribute relevance |
|  |  | Solar Energy greater than 500 W/m2 (Mars) | 4 |  |
|  |  | Solar Energy greater than 100 W/m2 (Ceres) | 3 |  |
|  |  | Solar Energy greater than 50W/m2 (Jupiter) | 2 |  |
|  |  | Solar not possible, but other sources available | 1 |  |
|  |  | Only nuclear (RTG, fission/fusion) available | 0 |  |
|  | Plant Growthac | Excellent – similar to fertile terrestrial soil | 5 | acScoring methodology assigned by the author based on comparative analysis of attribute relevance |
|  |  | Good – Basic growth possible with minimal nutrients and other support | 4 |  |
|  |  | Moderate – Plant growth possible but somewhat stunted or slow growing with nutrients | 3 |  |
|  |  | Poor – stunted growth possible with significant help (nutrients, fertilizers, etc.) | 2 |  |
|  |  | Negligible – not possible to grow or unknown | 1 |  |
|  |  | Not possible | 0 |  |
| Scientific benefits | Planetary geologyad | Exceptionally rich and diverse geology - provides a thorough record of planetary formation and evolution– e.g., Earth | 5 | adScoring methodology assigned by the author based on comparative analysis of attribute relevance |
|  |  | High scientific value with diverse geological features— both ancient and ongoing processes, surface renewal | 4 |  |
|  |  | Moderate geological diversity with important features (impact structures, cryovolcanism, some tectonic features, maria, polar diversity | 3 |  |
|  |  | Low levels of complexity or activity; surfaces are relatively static and offer only limited insights into planetary evolution. | 2 |  |
|  |  | No geologic diversity, mission data, or evidence of activity now or in the past | 1 |  |
|  | Planetary/astro-chemistryae | Exceptionally rich - complex organic and inorganic processes occur on a large scale | 5 | aeScoring methodology assigned by the author based on comparative analysis of attribute relevance |
|  |  | High chemical diversity with active or past reactions that yield a broad range of molecular chemistry | 4 |  |
|  |  | Moderate - A variety of types (e.g., simple organics, hydrated minerals, or past aqueous alteration products) | 3 |  |
|  |  | Low to modest chemical complexity—some simple organics or volatile reactions may occur, but the range is limited. | 2 |  |
|  |  | Trace or rudimentary chemical compounds are present; little evidence for active or diverse chemical processing | 1 |  |
|  | Astrobiology Potentialaf | Exceptionally high, The environment is active in ways that could readily support life—possibly hosting active ecosystems. | 5 | afScoring methodology assigned by the author based on comparative analysis of attribute relevance |
|  |  | High - evidence for a subsurface ocean with adequate energy (e.g., from tidal heating), and chemical ingredients | 4 |  |
|  |  | Moderate - Past or present liquid water (or an ocean) is indicated, and key chemical ingredients may be available | 3 |  |
|  |  | Low - Some evidence exists for a past/present liquid water, but overall conditions inhibit biological potential | 2 |  |
|  |  | Minimal - Only trace organics or minimal energy exist; any life would have extreme constraints | 1 |  |
|  |  | None - No liquid water or energy sources; the environment is entirely inhospitable (or nearly so). | 0 |  |
|  | Atmospheric science/climatologyag | Exceptionally complex or dense atmospheres with active meteorology and substantial chemistry. (Titan or Earth) | 5 | agScoring methodology assigned by the author based on comparative analysis of attribute relevance |
|  |  | A relatively thick atmosphere with active dynamics, seasonal cycles, and higher chemical diversity. | 4 |  |
|  |  | A thin atmosphere with fluidic properties that still has weather (e.g., dust storms) and moderate chemistry | 3 |  |
|  |  | A weak, thin atmosphere with limited seasonal activity. | 2 |  |
|  |  | Tenuous atmosphere with no fluidic properties, atomic/molecular level only | 1 |  |
|  |  | No atmosphere | 0 |  |
|  | Space physics/radiation environmentah | Exceptional - intense or complex radiation environments | 5 | ahScoring methodology assigned by the author based on comparative analysis of attribute relevance |
|  |  | High – Active radiation environment, ideal for studying solar and galactic particles in near‐vacuum conditions | 4 |  |
|  |  | Moderate - significant cosmic ray exposure and occasional SEP events; the environment is active but not extreme. | 3 |  |
|  |  | Low - radiation environment where GCR and SEP fluxes are present but moderated (by distance or modest shielding), | 2 |  |
|  |  | Minimal - only a weak or very low range of particle radiation, | 1 |  |
|  |  | Virtually no radiation environment of interest | 0 |  |
|  | Remote sensing observationsai | Exceptionally high - Near-planetary scale views with outstanding resolution and spectral diversity. | 5 | aiScoring methodology assigned by the author based on comparative analysis of attribute relevance |
|  |  | Good to High - quality, high-resolution imaging and multispectral data are available over a wide range. | 4 |  |
|  |  | Moderate - quality local data are achievable, revealing regional-scale features and some spectral diversity | 3 |  |
|  |  | Limited - Modest resolution and spectral data can be obtained, but observational scope is reduced due to low solar light | 2 |  |
|  |  | Very limited remote sensing capability | 1 |  |
|  |  | Almost no useful data can be obtained locally | 0 |  |
|  | Hydrologicaj | Exceptional - active hydrologic processes that include a regional or global cycle(s) | 5 | ajScoring methodology assigned by the author based on comparative analysis of attribute relevance |
|  |  | High – seasonal fluid cycle exists, with observed polar caps, and frost processes | 4 |  |
|  |  | Moderate - hydrologic activity is present or preserved in the geologic record; has transient atmospheres driven by sublimation/deposition, or evidence of cryovolcanism | 3 |  |
|  |  | Low - Trace or extremely limited evidence of ice or frost | 2 |  |
|  |  | Minimal - Some localized or transient evidence of volatile-related processes—such as seasonal frost or minimal sublimation | 1 |  |
|  |  | None - no fluid-related processes | 0 |  |
|  | Comparative planetologyak | Exceptional - the body offers a nearly unique window into one or several major evolutionary processes | 5 | akScoring methodology assigned by the author based on comparative analysis of attribute relevance |
|  |  | High - geologic, chemical, or other signatures make the body a strong reference point for understanding differentiation, volatile loss, and impact history | 4 |  |
|  |  | Moderate - distinct features (such as preserved ancient surfaces or evidence for water alteration) provide important, though not unique, constraints on planetary formation and evolution. | 3 |  |
|  |  | Low - while some evidence (e.g., cratering history) exists, it contributes only modestly to solar system evolution | 2 |  |
|  |  | Minimal - body has limited distinguishing features, yielding only basic evolutionary insights | 1 |  |
|  |  | None - no useful information on planetary evolution (e.g., it is too altered, homogenous) | 0 |  |

Appendix B. Attribute Desirability Scores for All Worlds

| Attribute | Moon | Mars | Ceres | Callisto | Titan | Oberon | Triton | Pluto | References |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Timescale to travel from Earth | 5 | 4 | 4 | 3 | 2 | 1 | 0 | 0 | Estimated from Thomas except for Pluto, which is ref in paper (2016) |
| Amount of fuel needed | 5 | 5 | 4 | 3 | 3 | 2 | 1 | 1 | Estimated from Thomas except for Pluto, which is ref in paper (2016) |
| Orbital Entry dV needed | 5 | 3 | 1 | 4 | 4 | 3 | 4 | 2 | Calculated (cmglee, 2024) |
| Orbit to landing dV needed | 3 | 2 | 5 | 1 | 2 | 3 | 2 | 4 | Calculated (cmglee, 2024) |
| Access to other bodies | 5 | 1 | 2 | 4 | 5 | 3 | 3 | 3 | Qualitative scoring based on standardly known planetary systems |
| Low Gravity Effects | 5 | 2 | 3 | 1 | 2 | 1 | 1 | 1 | NASA (2025) |
| Radiation Effects | 2a | 3b | 3c | 5d | 5e | 3d, f | 3d, f | 3d | aDixit, 2024; bHassler, 2023; cLetaw, 1988; dRingwald, 2008; eSittler, 2014; fUCB, 2024 |
| Temperature Extremes | 2g | 3h | 3i | 2j | 3k | 1l | 1m | 1n | gNASA-LRO, 2025; hNASA-MSL, 2025; iHille, 2023; jSquyres, 1980; kJennings, 2016; lHowett, 2023; mStansberry, 1992; nEarle, 2017 |
| Comm Delay | 5 | 3 | 2 | 2 | 2 | 1 | 1 | 1 | Based on speed of light and distance. |
| Seismic/Volcanic Hazards | 2o | 2p | 3q | 4r | 3s | 4t | 2u | 4v | oKim, 2024; pKutbunur, 2023; qBuckzowski, 2016; rMeye, 2024; sLopes, 2007); tBlanco-Rojas, uAadithiva, 2015); vHand, 2025 |
| Weather Hazards | 3 | 2 | 3 | 3 | 2 | 3 | 3 | 3 | Based on knowledge of known levels of atmosphere present |
| Meteorite Impacts | 1 | 2 | 1 | 1 | 3 | 1 | 1 | 1 | Based on knowledge of known levels of atmosphere present for protection from meteorites |
| Visibility/Low Light | 3 | 5 | 5 | 3 | 2 | 1 | 1 | 1 | Based on distance from sun, if thick atmosphere present, and length of night |
| Terrain Difficulty | 3 | 3 | 2w | 1x | 4y | 2z | 3aa | 3aa | Moon and Mars are well studied surfaces; wErmakov, 2019; xZahnle, 1998; yNASA/ESA-Huygens, 2024); zHelfenstein, 1991; 27aaSchenk, 2021 |
| Regolith Hazards | 1ab | 2ac | 3ad | 4ae | 3af | 4ag | 3ah | 2ai | abNASA-Apollo, et. al., 2025; acArcher, 2019; adRussel, 2016; aeGreeley, 2000; afNASA/ESA-Huygens, 2024); agNASA-Voyager 2, 2025; ahSmith, 1989; aiStern, 2015 |
| Mineral Content | 4aj | 3ak | 4al | 2am | 1an | 0ao | 0ap | 1aq | ajBadescu, 2012; akCarr, 2006; alBell, 2025; amLopes, 2019; anClark,, 2012; aoSchenk & Moore, 2020; apSchenk, 2021; aqMoore, 2016 |
| Water Availability | 2ar | 4as | 4at | 3at | 4au | 3av | 3aw | 3ax | arAnand, 2010; asDundas, 2018; atPrettyman, 2017; auBurger, 2007; avCruikshank & Brown, 1981; awCruikshank, 1993; axGrundy, 2016 |
| Atmospheric Resources | 3ay | 4az | 4ba | 3bb | 5bc | 2bd | 2be | 3bf | ayAnand, 2012;azMancinelli & Bannin, 2003; baDe Sanctis, 2024; bbHand, 2009; bcGajeri, 2021; bdCartwright, 2023; beCruikshank, 2023; bfBrown, 2002 |
| Helium-3 (Fusion Fuel) | 2bg | 0bh | 1bi | 1bj | 2bk | 3bl | 3bl | 0bm | bgBruhaug & Philips, 2021; bhOwen, 1988; biPrettman, 2017; bjRingwald, 2000; bkVan Cleve, 2005; blPalsewski, 2021; bmMcKinnon, 2020 |
| Deuterium (Fusion Fuel) | 2bn | 4bo | 3bp | 2bq | 3br | 3bs | 3bs | 2bt | bnBruhaug & Philips, 2021; boZubrin, 2025; bpPrettyman, 2017; bqMahaffy, 1998; brCoustenis, 2008; bsPalsewski, 2021; btMcKinnon, 2020 |
| Lithium (Fusion Fuel) | 2bu | 1bv | 3bw | 1bx | 1bx | 1bx | 1bx | 1bx | buZhai, 1996; bvMagna, 2013; bwStone, 2021; bxAnders & Grevesse, 1989 |
| Boron (Fusion Fuel) | 1by | 3bz | 1ca | 1ca | 1ca | 1ca | 1ca | 1ca | byZhai, 1996; bzNellesen, 2023; caAnders & Grevesse, 1989 |
| Building Materials | 3cb | 4cb | 2cc | 2cc | 1cc | 1cc | 1cc | 1cc | cbVitali, 2022; ccBritt, 2002 |
| Complex Chemistry | 2cd | 3ce | 3cf | 2cg | 5ch | 2ci | 4cj | 4ck | cdZhang & Paige, 2009; ceEigenbrode, 2018; cfKaplan, 2018; cgMcCord, 1997; chCable, 2012; ciCartwright, 2018; cjThompson, 1989; ckCruikshank, 2021 |
| Radioactive Elements | 4cl | 3cm | 2cl | 2cn | 1cn | 1cn | 1cn | 1cn | clSurkov & Fedoseyev, 1978; cmMarspedia, 2023; cnTye, 2017 |
| Alternative Energy Potential | 5co | 4co | 3co | 2co | 3co,cp | 0co,cq | 1co | 1co,cr | coBarth, 2004; cpWang, 2017; cqBlanco-Rojas, 2024; crHand, 2015 |
| Plant Growth | 3cs | 2ct | 1 | 1 | 1 | 1 | 1 | 1 | csKeeter, 2023;ct Temming, 2022; Rest are estimates – current information is unknown. |
| Planetary Geology | 4cu | 4cv | 3cw | 5cx | 5cy | 2cz | 4da | 4db | cuJaumann, 2012; cvZubrin, 1996; cwBuczowski, 2016; cxPrentice, 1999; cyLopes, 2007 & 2019; czHelfenstein, 1991; daSchenk, 2021; dbStern, 2015 |
| Astrochemistry | 1dc | 3dd | 3de | 4df | 5dg | 2dh | 3di | 3dj | dcZhang & Paige; 1996; ddEigenbroode, 2018; deDe Sanctis & Raponi, 2022; dfMcCord, 1997; dgCable, 2012; dhMoses, 2020; diThompson, 2016; djGrundy, 2016 |
| Atsrobiology | 1dk | 4dl | 3dm | 5dl | 5dl | 1dn | 2do | 2do | dkFeldman, 2001; dlDe Mol, 2023; dmCastillo-Rogez, 2020; dnWeber & Leonard, 2024; doHendrix, 2023 |
| Atmospheric/Meteorology | 0 | 3dp | 1 | 4 | 5dq | 4 | 4 | 2dr | dpNASA, 2025; dqTomasko, 2005; 1drVidmachenko, 2025. Rest are scored per standardly available information. |
| Space Physics/Rad | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | All scored similarly due to having their own unique, interesting radiation environments |
| Remote Sensing | 5 | 5 | 5 | 5 | 5 | 5 | 5 | 5 | All scored similarly due to having their own unique, interesting observation environments |
| Hydrologic | 1ds | 3dt | 2du | 3dv | 4dw | 1dx | 2dy | 2dy | dsAnand, 2010; dtMontmessin, 2024; duCastillo-Rogez, 2020; dvHand, 2009; dwTomasko, 2005; dxBlanco-Rojas, 2024; dySchenk, 2021 |
| Comparative Planetology | 4 | 4 | 3 | 5 | 5 | 2 | 4 | 4 | All scores based on authors knowledge, based on available information |
| Total | 98 | 108 | 96 | 99 | 111 | 71 | 76 | 74 | Sum total before normalization and weighting |

References

Aadithya TA, et al. Case study on human colonization of Triton; 2015. World Research Library. Accessed June 2025. https://www.worldresearchlibrary.org/up_proc/pdf/99-14483036648-10.pdf

American Chemical Society. Organic Chemistry; n.d. Accessed June 2025. https://www.acs.org/careers/chemical-sciences/areas/organic-chemistry.html

Anand M. Lunar water: A brief review. Earth Moon Planets. 2010;107(1):65-73. doi:10.1007/s11038-010-9377-9

Anand M, Crawford IA, Balat-Pichelin M, et al. A brief review of chemical and mineralogical resources on the Moon and likely initial in situ resource utilization (ISRU) applications. Planet Space Sci. 2012;74(1):42-48. doi:10.1016/j.pss.2012.08.012

Anders E, Grevesse N. Abundances of the elements: meteoritic and solar. Geochim Cosmochim Acta. 1989;53(1):197-214. doi:10.1016/0016-7037(89)90286-X

Archer PD Jr, Ming DW, Sutter B, et al. Perchlorate on Mars—overview and implications. NASA Technical Reports Server; 2019. Accessed June 2025. https://ntrs.nasa.gov/api/citations/20190028297/downloads/20190028297.pdf

Badescu V. Moon: Prospective Energy and Material Resources. Springer Science+Business Media; 2012.

Barth JL. Space and atmospheric environments: from low earth orbits to deep space. In: Kleiman JI, Iskanderova Z, eds. Protection of Materials and Structures from Space Environment. Kluwer Academic Publishers Publishers; 2004:7-29. doi:10.1007/1-4020-2595-5_2

Bell JF, Ravine MA, Caplinger MA, et al. The Psyche multispectral imager investigation: characterizing the geology, topography, and multispectral properties of a metal-rich world. Space Sci Rev. 2025;221(4):47. doi:10.1007/s11214-025-01169-3

Blanco-Rojas M, Sori MM, Pamerleau IF. Oberon as an ocean world? Insights from the topography of one of the tallest peaks in the solar system. LPI Contrib. 2024;3040:1468.

Bruhaug G, Phillips W. Nuclear fuel resources of the moon: A broad analysis of future lunar nuclear fuel utilization. NSS Space Settlement J. 2021;5.

Buckley A. Engineering in cold climates: freezing temps, ice & snow; 2024. Carroll Engineering Corporation. Accessed Jan 2024. https://www.carrollengineering.com/engineering-in-cold-climates-temperatures/

Buczkowski DL, Schmidt BE, Williams DA, et al. The geomorphology of Ceres. Science. 2016;353(6303):aaf4332. doi:10.1126/science.aaf4332

Burger MH, Sittler EC Jr, Johnson RE, Smith HT, Tucker OJ, Shematovich VI. Understanding the escape of water from Enceladus. J Geophys Res. 2007;112(A6). doi:10.1029/2006JA012086

Bussard R, Bussard R. System technical and economic features of qed-engine-driven space transportation. In: 33rd Joint Propulsion Conference and Exhibit; 1997:(3071). doi:10.2514/6.1997-3071

Cable ML, Hörst SM, Hodyss R, Beauchamp PM, Smith MA, Willis PA. Titan tholins: simulating Titan organic chemistry in the Cassini-Huygens era. Chem Rev. 2012;112(3):1882-1909. doi:10.1021/cr200221x

Carr MH. The Surface of Mars. Cambridge University Press; 2006.

Cartwright RJ, DeColibus RA, Castillo-Rogez JC, Beddingfield CB, Grundy WM, Nordheim TA. Are nitrogen-bearing species present on Uranus’ moon Umbriel? 54th Lunar and Planetary Science Conference. Vol 2806; 2023.

Cartwright RJ, Emery JP, Pinilla-Alonso N, Lucas MP, Rivkin AS, Trilling DE. Red material on the large moons of Uranus: dust from the irregular satellites? Icarus. 2018;314:210-231. doi:10.1016/j.icarus.2018.06.004

Castillo-Rogez JC, Neveu M, Scully JEC, et al. Ceres: astrobiological target and possible ocean world. Astrobiology. 2020;20(2):269-291. doi:10.1089/ast.2018.1999

Clark RN, Cruikshank DP, Jaumann R, et al. The surface composition of Iapetus: mapping results from Cassini VIMS. Icarus. 2012;218(2):831-860. doi:10.1016/j.icarus.2012.01.008

Clarke JD. Extraterrestrial regolith. Regolith Sci. 2009;1:377-407.

Cmglee. Solar system Delta-V map. Wikipedia; 2024. Accessed June 2025. https://en.wikipedia.org/wiki/Delta-v_budget#/media/File:Solar_system_delta_v_map.svg

Coustenis A, Jennings DE, Jolly A, et al. Detection of C2HD and the D/H ratio on Titan. Icarus. 2008;197(2):539-548. doi:10.1016/j.icarus.2008.06.003

Cruikshank DP, Hamilton Brown RH. The Uranian satellites: water ice on Ariel and Umbriel. Icarus. 1981;45(3):607-611. doi:10.1016/0019-1035(81)90026-9

Cruikshank DP, Hamilton Brown RH, Clark RN. Nitrogen on triton. Icarus. 1984;58(2):293-305. doi:10.1016/0019-1035(84)90046-0

Cruikshank DP, Roush TL, Owen TC, et al. Ices on the surface of Triton. Science. 1993;261(5122):742-745. doi:10.1126/science.261.5122.742

De Mol ML. Astrobiology in space: A comprehensive look at the solar system. Life (Basel). 2023;13(3):675. doi:10.3390/life13030675

Sanctis MCD, Raponi A. Ceres’ surface composition. In: Marchi S, Raymond CA, Russell CT, eds. Vesta and Ceres: Insights from the Dawn Mission for the Origin of the Solar System. Cambridge University Press; 2022:105-120. doi:10.1017/9781108856324.010

Dixit M. How space radiation threatens lunar exploration; 2023. Smithsonian Institution. Accessed June 2025. https://www.smithsonianmag.com/science-nature/how-space-radiation-threatens-lunar-exploration-180981415/

Dundas CM, Bramson AM, Ojha L, et al. Exposed subsurface ice sheets in the Martian mid-latitudes. Science. 2018;359(6372):199-201. doi:10.1126/science.aao1619

Earle AM, Binzel RP, Young LA, et al. Long-term surface temperature modeling of Pluto. Icarus. 2017;287:37-46. doi:10.1016/j.icarus.2016.09.036

Eigenbrode JL, Summons RE, Steele A, et al. Organic matter preserved in 3-billion-year-old mudstones at Gale crater, Mars. Science. 2018;360(6393):1096-1101. doi:10.1126/science.aas9185

Enss C, Hunklinger S. Low Temp Phys. Springer Science+Business Media. 2005.

Ermakov AI, Kreslavsky MA, Scully JEC, Hughson KHG, Park RS. Surface roughness and gravitational slope distributions of Vesta and Ceres. J Geophys Res Planets. 2019;124(1):14-30. doi:10.1029/2018JE005813

Feldman WC, Maurice S, Lawrence DJ, et al. Evidence for water ice near the lunar poles. J Geophys Res. 2001;106(E10):23231-23251. doi:10.1029/2000JE001444

Gajeri M, Aime P, Kezerashvili RY. A Titan mission using the direct fusion drive. Acta Astronaut. 2021;180:429-438. doi:10.1016/j.actaastro.2020.12.013

Goswami N, White O, Blaber A, Evans J, van Loon JJWA, Clement G. Human physiology adaptation to altered gravity environments. Acta Astronaut. 2021;189:216-221. doi:10.1016/j.actaastro.2021.08.023

Greeley R, Klemaszewski JE, Wagner R; Galileo Imaging Team. Galileo views of the geology of Callisto. Planet Space Sci. 2000;48(9):829-853. doi:10.1016/S0032-0633(00)00050-7

Grundy WM, Binzel RP, Buratti BJ, et al. Surface compositions across Pluto and Charon. Science. 2016;351(6279):1283.

Hand E. Pluto is alive—but where is the heat coming from? Science. 2015. Accessed June 2025. https://www.science.org/content/article/pluto-alive-where-heat-coming

Hand KP, Chyba CF, Priscu JC, Carlson RW, Nealson KH. Astrobiology and the Potential for Life on Europa. Vol 589. Europa Publications; 2009:629.

Hassler DM, Zeitlin C, Wimmer-Schweingruber RF, et al. Mars’ surface radiation environment measured with the Mars Science Laboratory’s Curiosity rover. Science. 2014;343(6169):1244797. doi:10.1126/science.1244797

Health Physics Society. Effects of radiation. Radiation Answers; n.d. Accessed May 2025. https://www.radiationanswers.org/radiation-and-me/effects-of-radiation.html

Helfenstein P, Hillier J, Weitz C, Veverka J. Oberon: color photometry from voyager and its geological implications. Icarus. 1991;90(1):14-29. doi:10.1016/0019-1035(91)90065-2

Hendrix AR, Hurford T, Patterson GW, et al. Potential ocean worlds white paper submitted to the 2023-2032 Planetary Science and Astrobiology Decadal Survey. NASA Technical Reports Server; n.d. Accessed June 2025. https://ntrs.nasa.gov/api/citations/20190000935/downloads/20190000935.pdf

Hille KB. Dawn maps Ceres craters where ice can accumulate; 2023. National Aeronautics and Space Administration. Accessed June 2025. https://www.nasa.gov/solar-system/dawn-maps-ceres-craters-where-ice-can-accumulate/

Howett CJA, Miles GM, Spencer JR. Constraints on observing the Uranian satellites in the infrared. Science. 2023;233:70-74.

Illuminating Engineering Society of North America (IESNA). Lighting Handbook: Reference & Application; 2011.

Jaumann R, Hiesinger H, Anand M, et al. Geology, geochemistry, and geophysics of the Moon: status of current understanding. Planet Space Sci. 2012;74(1):15-41. doi:10.1016/j.pss.2012.08.019

Jennings DE, Cottini V, Nixon CA, et al. Surface temperatures on Titan during northern winter and spring. ApJL. 2016;816(1):L17. doi:10.3847/2041-8205/816/1/L17

Johnson SK. New map shows the strange terrain of Titan. Ars Technica; 2019. Accessed June 2025. https://arstechnica.com/science/2019/11/new-map-shows-the-strange-terrain-of-titan/

Jones H. The partial gravity of the moon and mars appears insufficient to maintain human health. 50th International Conference on Environmental Systems; 2021.

Keeney RL. Creativity in decision making with value-focused thinking. MIT Sloan Manag Rev. 1994. Accessed June 2025. https://sloanreview.mit.edu/article/creativity-in-decision-making-with-valuefocused-thinking/

Keeter B. Scientists grow plants in lunar soil; 2023. National Aeronautics and Space Administration. Accessed June 2025. http://www.nasa.gov/humans-in-space/scientists-grow-plants-in-lunar-soil/

Kim SE. The moon is shrinking, causing moonquakes at a potential NASA landing site, study finds. Smithsonian Magazine; 2024. Accessed June 2025. https://www.smithsonianmag.com/smart-news/the-moon-is-shrinking-causing-moonquakes-at-a-potential-nasa-landing-site-study-finds-180983737/

Kittel C, McEuen P. Introduction to Solid State Physics. John Wiley & Sons; 2018

Letaw JR, Silberberg R, Tsao CH. Galactic cosmic radiation doses to astronauts outside the magnetosphere. In: McCormack PD, Swenberg CE, Bücker H, eds. Terrestrial Space Radiation and Its Biological Effects. Springer US; 1988:663-673. doi:10.1007/978-1-4613-1567-4_46

Lopes RMC, Mitchell KL, Stofan ER, et al. Cryovolcanic features on Titan’s surface as revealed by the Cassini Titan Radar Mapper. Icarus. 2007;186(2):395-412. doi:10.1016/j.icarus.2006.09.006

Lopes RMC, Wall SD, Elachi C, et al. Titan as revealed by the Cassini radar. Space Sci Rev. 2019;215(4):1-50. doi:10.1007/s11214-019-0598-6

Magna T, Day JMD, Mezger K, et al. Lithium isotope constraints on crust–mantle interactions and surface processes on Mars. Geochim Cosmochim Acta. 2015;162:46-65. doi:10.1016/j.gca.2015.04.029

Mahaffy PR, Donahue TM, Atreya SK, Owen TC, Niemann HB. Galileo probe measurements of D/H and 3He/4He in Jupiter’s atmosphere. Space Sci Rev. 1998;84(1-2):251-263. doi:10.1023/A:1005091806594

Mancinelli RL, Banin A. Where is the nitrogen on Mars? Int J Astrobiol. 2003;2(3):217-225. doi:10.1017/S1473550403001599

Marspedia. Radioactive rarity on Mars. Marspedia; 2023. Accessed June 2025. https://marspedia.org/Radioactive_Rarity_on_Mars

McCord TB, Carlson RW, Smythe WD, et al. Organics and other molecules in the surfaces of Callisto and Ganymede. Science. 1997;278(5336):271-275. doi:10.1126/science.278.5336.271

McKinnon WB, Glein CR, Bertrand T, Rhoden AR. Formation, composition, and history of the Pluto system: A post-New-Horizons synthesis. arXiv; 2020.

McNutt RL, Horsewood J, Fiehler DI. Human missions throughout the Outer Solar System: requirements and implementation. Johns Hopkins APL Tech Dig. 2010;28(4).

Montmessin F, Fedorova A, Alday J, et al. Mars’ water cycle and escape: A view from Mars Express and beyond. Space Sci Rev. 2024;220(7):77. doi:10.1007/s11214-024-01099-6

Moore JM, McKinnon WB, Spencer JR, et al. The geology of Pluto and Charon through the eyes of New Horizons. Science. 2016;351(6279):1284-1293. doi:10.1126/science.aad7055

Moses JI, Cavalié T, Fletcher LN, Roman MT. Atmospheric chemistry on Uranus and Neptune. Philos Trans A Math Phys Eng Sci. 2020;378(2187):20190477. doi:10.1098/rsta.2019.0477

Nagar B, Bardiya R, Bendale A, Danao K, Karande N. Boron: A micronutrient yet indispensable. In: Recent Trends in Botanical Sciences; 2022.

National Aeronautics and Space Administration (NASA). Lunar Reconnaissance Orbiter (LRO)—temperature variation on the Moon; 2014. Accessed June 2025. https://science.nasa.gov/wp-content/uploads/2024/01/lrolitho7temperaturevariation27may2014.pdf

National Aeronautics and Space Administration (NASA). NASA-STD-3001 Technical brief—Design for ionizing radiation protection; 2023. Accessed June 2025. https://www.nasa.gov/wp-content/uploads/2023/03/radiation-protection-technical-brief-ochmo.pdf

National Aeronautics and Space Administration (NASA). Jupiter moons; 2025. Accessed June 2025. https://science.nasa.gov/jupiter/jupiter-moons/

National Aeronautics and Space Administration (NASA). Neptune moons; 2025. Accessed June 2025. https://science.nasa.gov/neptune/moons/

National Aeronautics and Space Administration (NASA); 2025. Accessed June 2025. https://www.nasa.gov/feature/artemis/

National Aeronautics and Space Administration (NASA); 2025. Facts. Accessed June 2025. https://science.nasa.gov/mars/facts/

National Aeronautics and Space Administration (NASA). Is there a lunar meteorite hazard? Space Math at NASA; n.d. Accessed June 2025. https://spacemath.gsfc.nasa.gov/moon/5Page22.pdf

Nellessen MA, Gasda P, Crossey L, et al. Boron adsorption in clay minerals: implications for Martian groundwater chemistry and boron on Mars. Icarus. 2023;401:115599. doi:10.1016/j.icarus.2023.115599

Nguyen N, Kim G, Kim K-S. Effects of microgravity on human physiology. Korean J Aerosp Environ Med. 2020;30(1):25-29. doi:10.46246/KJAsEM.30.1.25

Nordheim TA, Castillo-Rogez JC, Villarreal MN, Scully JEC, Costello ES. The radiation environment of Ceres and implications for surface sampling. Astrobiology. 2022;22(5):509-519. doi:10.1089/ast.2021.0080

Owen T, Maillard JP, De Bergh C, Lutz BL. Deuterium on Mars: the abundance of HDO and the value of D/H. Science. 1988;240(4860):1767. doi:10.1126/science.240.4860.1767

Prentice AJR. Origin, bulk chemical composition and physical structure of the Galilean satellites of Jupiter: A post-Galileo analysis. Earth Moon Planets. 1999;87(1):11-55. doi:10.1023/A:1010692812892

Prettyman TH, Yamashita N, Toplis MJ, et al. Extensive water ice within Ceres’ aqueously altered regolith: evidence from nuclear spectroscopy. Science. 2017;355(6320):55-59. doi:10.1126/science.aah6765

Review of the universe. Nucl-Helium. n.d.;3. Accessed Apr 2025. https://universe-review.ca/F14-nucleus10.htm

Ringwald FA. SPS 1020 introduction to space sciences; 2000. California State University – Fresno. Accessed June 2025. https://zimmer.fresnostate.edu/~fringwal/w08a.jup.txt

Royal Museums Greenwich. Solar system data; n.d. Accessed June 2025. https://www.rmg.co.uk/stories/space-astronomy/solar-system-data

Russell CT, Raymond CA, Ammannito E, et al. Dawn arrives at Ceres: exploration of a small, volatile-rich world. Science. 2016;353(6303):1008-1010. doi:10.1126/science.aaf4219

Schenk PM. The geology of Callisto. J Geophys Res. 1995;100(E9):19023-19040. doi:10.1029/95JE01855

Schenk PM, Moore JM. Topography and geology of Uranian mid-sized icy satellites in comparison with Saturnian and Plutonian satellites. Philos Trans A Math Phys Eng Sci. 2020;378(2187):20200102. doi:10.1098/rsta.2020.0102

Schenk PM, Beddingfield CB, Bertrand T, et al. Triton: topography and geology of a probable ocean world with comparison to Pluto and Charon. Remote Sens. 2021;13(17):3476. doi:10.3390/rs13173476

Sittler EC, Cooper JF. Galactic cosmic ray (GCR) model of Titan and formation of HCNO exobiological molecules. American Astronomical Society Meeting Abstracts. 2014;223.

Squyres SW. Surface temperatures and retention of H2O frost on Ganymede and Callisto. Icarus. 1980;44(2):502-510. doi:10.1016/0019-1035(80)90040-8

Stansberry JA, Yelle RV, Lunine JI, McEwen AS. Triton’s surface-atmosphere energy balance. Icarus. 1992;99(2):242-260. doi:10.1016/0019-1035(92)90144-V

Stern SA, Bagenal F, Ennico K, et al. The Pluto system: initial results from its exploration by New Horizons. Science. 2015;350(6258):aad1815. doi:10.1126/science.aad1815

Stone M. Where in the galaxy will we mine lithium? Sci Fiction. 2021. Accessed June 2025. https://www.sciof.fi/lithium-beyond-earth/

Surkov YA, Fedoseyev GA. Radioactivity of the Moon, Planets, and Meteorites. NASA Technical Report No. 19780004993; 1978. Accessed June 2025. https://ntrs.nasa.gov/api/citations/19780004993/downloads/19780004993.pdf

Szlugaj J, Radwanek-Bąk B. Lithium sources and their current use. Miner Resour Manag. 2022;38(1):61-88. doi:10.24425/gsm.2022.140613

Thomas SJ, Paluszek M, Cohen S, McGreivy N, Evans E. Fusion-enabled Pluto orbiter and lander. In: AIAA Space and Astronautics Forum and Exposition. American Institute of Aeronautics and Astronautics; 2017:(5276). doi:10.2514/6.2017-5276

Thompson WR, Singh SK, Khare BN, Sagan C. Triton: stratospheric molecules and organic sediments. Geophys Res Lett. 1989;16(8):981-984. doi:10.1029/GL016i008p00981

Tomasko MG, Archinal B, Becker T, et al. Rain, winds and haze during the Huygens probe’s descent to Titan’s surface. Nature. 2005;438(7069):765-778. doi:10.1038/nature04126

Tye AM, Milodowski AE, Smedley PL. Distribution of natural radioactivity in the environment. British Geological Survey Internal Report; 2017. Accessed June 2025. https://earthwise.bgs.ac.uk/index.php/OR/17/001_The_distribution_of_natural_radioactivity_in_rocks

United States Department of Energy. DOE explains … deuterium-tritium fusion fuel; n.d. Accessed June 2025. https://www.energy.gov/science/doe-explainsdeuterium-tritium-fusion-fuel

Van Cleve JE, Grillmair C, Hanna M, Reinert R. Helium-3 mining aerostats in the atmospheres of the outer planets. 2nd International Planetary Probe Workshop; 2005. Accessed June 2025. https://ntrs.nasa.gov/citations/20070014644

Vidmachenko A. Changes in temperature and pressure in the atmosphere of Pluto. In: Collection of Scientific Papers «ΛΌГOΣ». European Scientific Platform; 2025:208-214. doi:10.36074/logos-07.03.2025.042

Wang B. Titan moons has a variety of useful energy sources for colonists. NextBigFuture; 2017. Accessed June 2025. https://www.nextbigfuture.com/2017/07/titan-moons-has-a-variety-of-useful-energy-sources-for-colonists.html

Weber JM, Leonard EJ. The astrobiological potential of the Uranian moon system. Astrobiology. 2024;24(8):839-844. doi:10.1089/ast.2024.0045

Williams CH. Nuclear fusion space propulsion research, experimentation, theory development, and systems analysis efforts led by the NASA Glenn Research Center (1994-2004). Ascend; 2020. Accessed June 2025. https://ntrs.nasa.gov/api/citations/20205007513/downloads/Williams%20ASCEND%20paper%202%209-11-2020.pdf

Zahnle K, Dones L, Levison HF. Cratering rates on the Galilean satellites. Icarus. 1998;136(2):202-222. doi:10.1006/icar.1998.6015

Zeitlin C, Hassler DM, Cucinotta FA, et al. Measurements of energetic particle radiation in transit to Mars on the Mars Science Laboratory. Science. 2013;340(6136):1080-1084. doi:10.1126/science.1235989

Zhai M, Nakamura E, Shaw DM, Nakano T. Boron isotope ratios in meteorites and lunar rocks. Geochim Cosmochim Acta. 1996;60(23):4877-4881. doi:10.1016/S0016-7037(96)00338-9

Zhang JA, Paige DA. Cold-trapped organic compounds at the poles of the Moon and Mercury: implications for origins. Geophys Res Lett. 2009;36(16). doi:10.1029/2009GL038614

Zubrin R. The case for colonizing Mars; 1996. National Space Society. Accessed June 2025. https://nss.org/the-case-for-colonizing-mars-by-robert-zubrin/